|  | K. J. Somaiya College of Engineering, Mumbai-77 (A Constituent College of Somaiya Vidyavihar University) Department of Electronics Engineering |  |
| --- | --- | --- |

| Course Name: | Analog Integrated Circuits and Applications | Semester: | VI |
| --- | --- | --- | --- |
| Date of Performance: | Jan 15, 2024 | Batch No: | A2 |
| Faculty Name: | Prof. Ashwini Kumar | Roll No: | 16010221042 |
| Faculty Sign & Date: |  | Grade/Marks: | /25 |

Experiment No: 2 
Title: To study Inverting and Non Inverting Amplifier 		using Op-amp

Aim and Objective of the Experiment:

To study Inverting and Non-Inverting Amplifier using op-amp 741

COs to be achieved:

CO2: Design circuits using op-amps as linear applications.

Theory:

The op amp used is high performance monolithic one constructed using the fair child planar epitaxial process. It is intended for wide range of analog applications. High common mode voltage & absence of latch up tendencies make 741 ideal for use as a Voltage follower. Important features are: 1. No frequency compensation required 2. No short circuit protection 3. Offset voltage nullifying capability 4. No latch up problem 5. Large common mode & differential gain

Circuit Diagram:

A)Inverting Amplifier:

| AICA Lab | Semester: VI | Academic Year: 2022-23 Roll No: |
| --- | --- | --- |

|  | K. J. Somaiya College of Engineering, Mumbai-77 (A Constituent College of Somaiya Vidyavihar University) Department of Electronics Engineering |  |
| --- | --- | --- |

B)Non- Inverting Amplifier:

Stepwise-Procedure:

A. Inverting and Non-inverting amplifier 1) Connect the circuit for Inverting configuration. 2) Apply sine wave input of 1Vp-p, 1kHz and measure output voltage. 3) Draw the input and output waveforms Vs time. 4) Increase input voltage from zero to maximum such that output goes in to saturation. 5) Measure VOH and VOL and draw transfer characteristic. 6) Repeat steps 2 to 5 for Non Inverting Amplifier

Observation Table:

| AICA Lab | Semester: VI | Academic Year: 2022-23 Roll No: |
| --- | --- | --- |

|  | K. J. Somaiya College of Engineering, Mumbai-77 (A Constituent College of Somaiya Vidyavihar University) Department of Electronics Engineering |  |
| --- | --- | --- |

A)Inverting amplifier: For Vin sine wave 1kHz Sr. No.; Vin volts(p-p); Vout volts(p-p); Gain (Vo/Vin). 1.; 1; 10; 10. 2.; 2; 17.5; 8.75. 3.; 1.5; 14; 9.3. Find out maximum peak to peak voltage (Vin) such that no clipping is observed. Increase input voltage to observe clipping and measure VOH and VOL B)Non-inverting amplifier: For Vin sine wave 1kHz Sr. No.; Vin volts(p-p); Vout volts(p-p); Gain (Vo/Vin). 1.; 1; 10; 10. 2.; 1.5; 15; 10. 3.; 2; 17; 8.2.

Calculation:

Design:- 𝑅𝐹𝑅3 𝑉𝑜𝑢𝑡= ((𝑅2∗𝑉𝑎) −(𝑅1∗𝑉𝑏 ))…………………………..(1) 1)To design Subtractor, 𝑅1 = 𝑅2 = 𝑅3 = 𝑅𝐹 So that equation 1 becomes 𝑉𝑜𝑢𝑡= (𝑉𝑎−𝑉𝑏) Assume, 𝑅1 = 𝑅2 = 𝑅3 = 𝑅𝐹= 10𝐾Ω

Results:

Inverting:

| AICA Lab | Semester: VI | Academic Year: 2022-23 Roll No: |
| --- | --- | --- |

|  | K. J. Somaiya College of Engineering, Mumbai-77 (A Constituent College of Somaiya Vidyavihar University) Department of Electronics Engineering |  |
| --- | --- | --- |

Non-Inverting:

Post Lab Questions:

| AICA Lab | Semester: VI | Academic Year: 2022-23 Roll No: |
| --- | --- | --- |

|  | K. J. Somaiya College of Engineering, Mumbai-77 (A Constituent College of Somaiya Vidyavihar University) Department of Electronics Engineering |  |
| --- | --- | --- |

1.List all linear applications of op-amp. Voltage follower (Unity gain amplifier) Inverting amplifier Non-inverting amplifier Summing amplifier Difference amplifier Integrator Differentiator Instrumentation amplifier Active filters 2.Design averaging circuit for four DC inputs. 3.Design Adder-Subtractor circuit using opamp. 𝑉𝑜𝑢𝑡= (𝑉3 + 𝑉4) −(𝑉1 + 𝑉2)

Conclusion:

In conclusion, the experiment aimed at implementing inverting and non-inverting amplifiers using

| AICA Lab | Semester: VI | Academic Year: 2022-23 Roll No: |
| --- | --- | --- |

|  | K. J. Somaiya College of Engineering, Mumbai-77 (A Constituent College of Somaiya Vidyavihar University) Department of Electronics Engineering |  |
| --- | --- | --- |

the IC 741 operational amplifier was successfully conducted. The key characteristics of operational amplifiers, such as high open-loop gain, differential inputs, and single-ended output, were demonstrated in both configurations

Signature of faculty in-charge with Date:

| AICA Lab | Semester: VI | Academic Year: 2022-23 Roll No: |
| --- | --- | --- |